Smart charger cabinets

MQTT protocol proposal

v0.7

Version Record:

| Serial number | Protocol number | Version number | Modify the content | Modifier | Update time |
| --- | --- | --- | --- | --- | --- |
| 1 | V101 | V101 |  |  |  |
| 2 | V101 | V101 | Revise: - Added"Events notification" topic key: value in Event - Blue mark used |  |  |
| 3 | V101 | V101 | Revise: 1.Revise MQTT's BMS information content based on BMS information |  |  |
| 4 | V101 | V101 | Revise: 1.Sample error change |  |  |
| 5 | V101 | V101 | Revise: 1.Add soc in “notification” |  |  |
| 6 | V101 | V102 | Revise: 1. Add threshold configuration topic publication and threshold configuration subscription response |  |  |
| 7 | V101 | V102 | 1. Add some events to the notification topic |  |  |
| 8 | V101 | V103 | 1. Add QR code address configuration |  |  |

Emphasize:

- MQTT protocol support version

V3.1/V3.1.1

- QoS class：2

Table of contents

Workflow representation	4

1.1. Sequence diagram for the process of saving cabinets data to our database	4

1.2. Sequence diagram for the battery charging operation	6

Topics description	7

2.1. Cabinet information	7

2.2. Open Slot	10

2.3. Events notification	11

2.4. Alerts	12

2.6.	Thresholds	12

2.7.	ThresholdsResponse	13

2.8.	Order information	14

2.9.	Order information confirmation	15

2.10.	Open slot request	15

Extra information	16

3.1. Slot description	16

Questions to the supplier	16

Workflow representation

1.1. Sequence diagram for the process of saving cabinets data to our database

1.2. Sequence diagram for the battery charging operation

1.3 Order Information Flowchart

Topics description

2.1. Cabinet information

Topic where the cabinet will send the complete status of the cabinet and the batteries inside of it every 30s and after a new battery is inserted or removed from one of the slots.

topic: /stations/info/{cabinet_mac_addr}

| Field name | Type | Description |
| --- | --- | --- |
| mac_addr | String | cabinet’s mac address |
| device_id | String | Device ID |
| ctr_pro | String | Ctr Board Protocol |
| ctr_softver | String | Ctr Board Software version |
| ctr_warning | BOOL | true:cabinet exist warning false:No warning |
| env_temperature | Integer | Environment temperature |
| empty_slots | Integer | Total empty slots in cabinet |
| total_slots | Integer | Total slots in cabinet |
| slots | List<Slot> | Information about individual slots (3.1. Slot description) |
| timestamp | DateTime | timestamp of message creation |

Example:

{

"mac_addr": "00-88-14-4D-4C-FB",

"device_id": "2600401516050001",

"ctr_pro": "V101",

"ctr_softver": "V101",

"ctr_warning": false,

"env_temperature": 24,

"empty_slots": 2,

"total_slots": 8,

"slots": [

{	//This slot has the battery charging and the relay is on

"sub_pro": "V101",

"sub_softver": "V101",

"id": 1,

"slot_temp"：34,

"slot_type"：1,		//battery slot

"door_status": 0,      //close

"sub_Exit_Err": false,

"order_num": "ORD202307111850123456",

"charging_volt": 120.1,

"charging_curr":2.3,

"charge_status":1,   //charging

"start_time":1690192606,

"end_time":1690192696

"deposit_time":0

},

{ 	//This slot is empty and the relay is disconnected

"sub_pro": "V101",

"sub_softver": "V101",

"id": 2,

"slot_temp"：33,

"slot_type"：0,    		//empty slot

"door_status": 0,   	//close

"sub_Exit_Err": false,

"order_num": "",

"charging_volt": 0.0,

"charging_curr":0.0,

"charge_status":0,   //uncharge, idle

"start_time":0,

"end_time":0

"deposit_time":0

},

{ 	//This slot has the battery, but charge end, and the relay is disconnected

"sub_pro": "V101",

"sub_softver": "V101",

"id": 3,

"slot_temp"：32,

"slot_type"：1,    		//battery slot

"door_status": 0,   	//close

"sub_Exit_Err": false,

"order_num": "",

"charging_volt": 0.0,

"charging_curr":0.0,

"charge_status":2,   //charge end

"start_time":1690192606,

"end_time":1690192906

"deposit_time":1690202606

}

{ 	//This slot is error and the relay is disconnected

"sub_pro": "V101",

"sub_softver": "V101",

"id": 4,

"slot_temp"：32,

"slot_type"：0,    		//empty slot

"door_status": 0,   	//close

"sub_Exit_Err": true,  //Error

"order_number": "",

"charging_volt": 0.0,

"charging_curr":0.0,

"charge_status":0,   //uncharge, idle

"start_time":0,

"end_time":0

"deposit_time":0

}

],

"timestamp": "2021-08-03 17:04:54.123Z"

}

2.2. Open Slot

Topic where the cabinet will receive the command to open specific slots.

topic: /stations/open_slot/{cabinet_mac_addr}

| Field name | Type | Description |
| --- | --- | --- |
| slot_id | Integer | Slot identification |
| slot_info | integer | 0x01:open empty slot door. Server 0x02:open battery slot door. Server 0x03:start charging, relay is on Cabinet/ Server 0x04:stop charging, relay is disconnected Cabinet/ Server 0x05:set battery slot to empty slot Server |
| order_number | String | business order number |
| timestamp | DateTime | timestamp of message creation |

Note:

Example:

{

"slot_id": 1,

"slot_info": 1,

"order_number": "ORD202307111945345678",

"timestamp": "2021-08-03 17:04:54.123Z"

}

2.3. Events notification

Topic where the cabinet will send notifications after every action it takes (for example: closing or opening a slot).

topic: /stations/notifications/{cabinet_mac_addr}

| Field name | Type | Description |
| --- | --- | --- |
| slot_id | Integer | Slot identification |
| order_number | String | business order number |
| event | String | name of the event. (open_slot, close_slot,open_slot_busy,open_slot_success,open_slot_sub_no_execution,door_err,sub_execution_timeout,sub_offline,open_empty_slot_bat_online,open_bat_slot_bat_offline,insert_faulty_battery,close_empty_no_save_bat,opt_success,close_slot_timeout,waiting_open_full_timeout, ota_upgrading，ota_sub_upgrading, ) |
| timestamp | DateTime | timestamp of message creation |

Example:

{

"slot_id": 1,

"order_number": "ORD202307111945345678",

"event": "open_slot",

"timestamp": "2021-08-03 17:04:54.123Z"

}

2.4. Alerts

Topic where the cabinet will send alerts if it identifies any software or hardware alerts or errors.

topic: /stations/alerts/{cabinet_mac_addr}

| Field name | Type | Description |
| --- | --- | --- |
| slot_id | Integer | Slot identification (null if the alert isn’t for a specific slot) |
| type | String | message type (alert, error) |
| message | String | message describing the problem (Error:)(cabinet_alert:"Lightning warning",” Smoke alarm ”,"Subsequent updates for the remaining labels";"Internal battery failure" charger_type_err:"Charger temperature is too high","Subsequent updates for the remaining labels") |
| timestamp | DateTime | timestamp of message creation |

Example:

{

"slot_id": 1,

"type": "Error",

"message": "Charging failure",

"timestamp": "2021-08-03 17:04:54.123Z"

}

  - Thresholds

The topic can be used for subscription thresholds and update thresholds. It is mainly used for debugging and initial production configuration. It is not recommended to publish the topic during use, because the service will restart after receiving the topic

topic: /stations/thresholds/{cabinet_mac_addr}

| Field name | Type | Description |
| --- | --- | --- |
| info_up_period | Integer | cabinet information reporting cycle（unit:sec） |
| max_charge_current | Integer | Maximum charging current value (unit:mA) |
| alert_up_period | Integer | alert reporting cycle (unit:sec) |
| slot_over_temp | Integer | when this value is reached, the slot will be powered off |
| over_temp | Integer | cabinet over temperature threshold, when this value is reached, the entire cabinet will be powered off |
| dev_id | String | Device Unique Identification |
| ad_switch | boolean | true:turn on screen false:turn off screen |
| open_times | String | Card Swipe Auto-Open Time (hh:MM) — 24-Hour Format default:00:00 |

Example:

{

"info_up_period": 60,

"max_charge_current"：20000,

"alert_up_period":300,

"slot_over_temp":80,

"over_temp":70,

"dev_id":"DEV888676767",

“ad_switch”:true,

“open_times”:”06:42”

}

  - ThresholdsResponse

It is mainly used to confirm whether the threshold is successfully set to the cabinet

topic: /stations/thresholds_response/{cabinet_mac_addr}

| Field name | Type | Description |
| --- | --- | --- |
| info_up_period | BOOL | true:success false:failed |
| max_charge_current | Bool | true:success false:failed |
| alert_up_period | Bool | true:success false:failed |
| slot_over_temp | Bool | true:success false:failed |
| over_temp | Bool | true:success false:failed |
| dev_id | Bool | true:success false:failed |
| ad_switch | bool | true:success false:failed |
| open_times | bool | true:success false:failed |

Example:

{

"info_up_period":true,

"max_charge_current"：true,

"alert_up_period":true,

"slot_over_temp":true,

"over_temp":true,

"dev_id":true,

“ad_switch”:true,

“open_times”:true

}

  - Order information

The terminal machine should promptly upload a record of the end of charging order for a certain slot and wait for the backend to respond to the charging order with that order number.

topic: /stations/order_info/{cabinet_mac_addr}

| Field name | Type | Description |
| --- | --- | --- |
| mac | string | cabinet’s mac address |
| slot_id | int | slot identification |
| order_num | string | business order number |
| start_time | int | local time when the slot is successfully opened |
| end_time | int | local time when charging ends due to full or failed charging |
| charge_time | int | charging duration, counting from opening the slot(unit:minutes) |
| charge_status | Int | 0:open empty slot 1:charging 2:finished 3:open full slot |
| deposit_time | Int | charge over start counting,end in open full slot |

  - Order information confirmation

The server must reply to the terminal's charging end record upload

topic: /stations/order_info_confirm/{cabinet_mac_addr}

| Field name | Type | Description |
| --- | --- | --- |
| mac | string | cabinet’s mac address |
| slot_id | int | slot identification |
| order_num | string | business order number |

Extra information

3.1. Slot description

| Field name | Type | Description |
| --- | --- | --- |
| sub_pro | String | Sub Board Protocol |
| sub_softver | String | Sub Board Software version |
| id | Integer | Slot identification |
| sub_Exit_Err | BOOL | true:sub exist warning false:No warning |
| slot_type | Integer | 0:emtpy slot; 1:battery slot; |
| door_status | Integer | 0：close; 1：open |
| slot_temp | Integer | Environmental temperature of a single slot |
| order_num | String | business order number |
| charging_volt | double | Charging voltage(unit:0.1V) |
| charging_curr | double | Charging current(unit:0.1A) |
| charge_status | Integer | 0:uncharge 1:charging 2:charge end |
| order_num | String | business order number |
| start_time | Integer | local time when the slot is successfully opened |
| end_time | Integer | local time when charging ends due to full or failed charging |
| deposit_time | Integer | charge over start counting,end in open full slot |

Questions to the supplier

- Do the cabinets have a unique ID other than the Mac Address?